Written Safety Procedures

- Doctor always asks female patient to verify non-pregnancy before taking X-ray exposure.

- Use of Gonad shields.

- Always remove other people in X-ray room (only patient is present) when taking X-rays.

- Only licensed dentists or qualified auxiliary personnel take films in our office.

- D.E.P. Notice to Employees present on the wall in development room.

- Increase the distance of the individual from the source.

- Reduce the duration of exposure using high-speed screens and film.

- Use protective barriers between the individual and the source.

- Use a 2.0 mm aluminum filter installed at the tube house port to remove the softer, less energetic portion of the x-ray beam.  This lowers the amount of scatter produced and decreases the exposure to the patient.