Written Safety Procedures
1. Doctor always asks female patient to verify non-pregnancy before taking X-ray exposure.

2. Use of Gonad shields.

3. Always remove other people in X-ray room (only patient is present) when taking X-rays.

4. Only licensed dentists or qualified auxiliary personnel take films in our office.

5. D.E.P. Notice to Employees present on the wall in development room. 

6. Increase the distance of the individual from the source.

7. Reduce the duration of exposure using high-speed screens and film.

8. Use protective barriers between the individual and the source.

9. Use a 2.0 mm aluminum filter installed at the tube house port to remove the softer, less energetic portion of the x-ray beam.  This lowers the amount of scatter produced and decreases the exposure to the patient. 
